Wind Power Economics

Mark E. Hardwidge

Rachael Jabusch

Wind power has many advantages for society, including environmental friendliness and reduced reliance on foreign resources.

Ultimately, however, economics is the primary factor determining support for most projects.  If wind power is “too expensive” it will receive little support.  Once it becomes a net positive (that is, it generates a profit), it becomes a rational short term investment, and levels of adoption should increase rapidly.

It appears that wind power is reaching this transition point, not because of increased environmental concern, but because simple economics now make wind turbines good investments.

In order to secure funding (on the order of several million dollars), it is also important for a project to not only be a good investment, but for this to be demonstrated to potential investors.

There are several factors influencing the economics of wind turbines:

- Initial cost of the turbine, construction, connection to the power grid, and so on.

- Recurring maintenance costs

- Cost of capital (what interest rate is paid on the loan, or what rate could we receive if we invested or money elsewhere)

- Inflation rate

- Cost of electricity

- Government ecological (“green”) incentives, which could reduce initial costs

Investors care about a number of things, including:

- Return on Investment (RoI) – How much (usually in %/yr) does the investment return to investors.

- Payback time – How long does it take for the income stream (initial costs and future profits) to turn to a net positive.  A long time period increases the risk of the project.

- Risk – How likely is it that the project will, in fact, return a profit.  Numerous factors contribute to this risk, ranging from changes in electricity prices to physical damage to the wind turbine

As a scoping study, we constructed an Excel model to determine the RoI of various scenarios, and investigated to the sensitivity to changes in various inputs to assess risk.

This model also serves to verify the model at http://www.windpower.org/en/tour/econ/econ.htm

Market(ing) Opportunities

Champaign County has a population of approximately 150,000 people.  We can approximate, therefore, that the total electrical power requirement of the nearby area is on the order of a few hundred megawatts.  This, however, is more than sufficient demand to purchase the entire supply of even a very large wind farm.

There are numerous ways in which one could pursue a profit from wind power management.

Complete management: A company could purchase (or lease) a small footprint of land on Yankee Ridge and one or more wind turbines, connect to the electrical grid, and sell the power.

This option, however, seems more complicated to start.

Another option would be to have a localized wind turbine / farm with a single customer.  Champaign-Urbana has several customers that could purchase 100% of the power supplied.  The University of Illinois, of course, uses many megawatts of power.  However, there are also apartment complexes with many, many units and considerable empty space.

Baytowne Apartments (on North Prospect), for instance, has approximately 500 apartments, an 800 acre fenced area, and considerable empty field space surrounding it.

As an extra bonus, they area somewhat close to the ridge of high ground in NW Champaign county.

It is worth investigating this, because the 500 apartments must use approximately 500 kW of power.  Two ‘small’ (~225 kW) wind turbines would be a lovely addition to the community, and might even raise the value of the property, because already-affluent renters might be happy to pay a higher rent for ‘environmentally friendly’ power.

There are numerous apartment complexes on the outskirts of Champaign-Urbana that appear to have similar situations.

A third option is manufacturing windmills for direct sale.  Due to the high demand and low supply, this could be a very lucrative business.  (However, because this paper is primary about the economics of producing and selling electricity, this idea is outside the scope.)

General Results for Economic Calculations

Although, as described above, the value of a wind turbine depends enormously on several factors, we selected ‘reasonable’ estimates for these values to calculate a ‘reasonable’ return on investment.

Based on current prices for electricity and wind turbines, a wind turbine produces a return on investment of approximately 9% (wholesale of electricity at 5c/kWhr) to 17% (replacement cost of electricity at 8c/kWhr).

A RoI of 17% vastly exceeds the cost of capital, (perhaps as high as 8% for a somewhat risky venture like a wind farm), and thus would be a very good investment.

In addition, because a wind turbine “only” costs about $1000/kW, a small, 225kW windmill could be purchased and installed for only a quarter million dollars.  This would pay for itself in only about seven years.  This payback period is extremely important because the longer the period, the greater the risk.

After this seven-year payback, (or even before, as the economic viability of the first turbine has been proven), it would be simple to add a second (or third, or fourth) turbine to the wind farm.

Therefore, it appears that the two most viable options for a “Champaign-Urbana Wind Power Company” would be either selling to the grid, or selling to a single customer.  Overall, there is a large potential for profit from the sale of wind power even in an such as central Illinois with a relatively low average wind speed.

Suggestions:

- Start small.  Starting small would allow many enormous advantages.  First, it would be easier to start and would have far less risk.  This makes is vastly more likely that it really would start.  Also, expansion would be vastly simpler once there was some sort of operation going.

- Opportunities in the area must be investigated, ideally with the assistance of business/marketing people.

- It is important to start soon to become the leader in the area.  Starting soon vastly exceeds the need to ‘start big’.

- Starting at all would instantly make us the leader in the local area.